Designing an Acid-Base Titration

Follow the link below to the Designing an Acid-Base Titration module.

https://dthdpu.shinyapps.io/TitrationApp/

- Click on the Investigation 1 tab at the top of the page.

  - Describe the effect of adjusting the initial [analyte] on the titration curve using the [titrant] = initial [analyte] setting.

  - Describe the effect of adjusting the initial [analyte] on the titration curve after you deselect the [titrant] = initial [analyte] setting.

  - Describe the effect of adjusting the analyte initial volume keeping all other variables the same.

  - Set the initial [analyte] and [titrant] = 0.10 M, and initial volume = 50.0 mL.  Determine which indicators would be effective, with minimal titration error, for determination of the end point.

  - Now select strong base for the analyte, and adjust the variables to see how the titration curve varies.

- Click on the Investigation 2 tab at the top of the page.

  - Describe the effect of changing the pKa of the weak acid.

  - Describe the effect of adjusting the initial [analyte] on the titration curve using the [titrant] = initial [analyte] setting.

  - Describe the effect of adjusting the initial [analyte] on the titration curve after you deselect the [titrant] = initial [analyte] setting.

  - Describe the effect of adjusting the analyte initial volume keeping all other variables the same.

  - Set the pKa = 5, the initial [analyte] and [titrant] = 0.10 M, and initial volume = 50.0 mL.  Determine which indicators would be effective, with minimal titration error, for determination of the end point.

  - Now select weak base for the analyte, and adjust the variables to see how the titration curve and indicator selection varies.

  - You are given a 5.0 mM solution of 2,4-dinitrophenol (pKa = 4.114).  Design a titration for which you can observe the end point within a 50 mL titrant range.  Select an initial [analyte], initial analyte volume, [titrant], and indicator.

- Click on the Investigation 3 tab at the top of the page.

  - Use the module to help you design a titration for maleic acid (cis-butenedioic acid) for which you can observe both end points within a 50 mL titrant range.  Select an initial [analyte], initial analyte volume, [titrant], and an indicator for each of the 2 end points.

Harvey, David.  “Designing an Acid-Base Titration.”  Analytical Sciences Digital Library.

http://community.asdlib.org/activelearningmaterials/designing-an-acid-base-titration/