GC columns

Learning Objectives

All students are expected to:

- Differentiate different types of columns (i.e., WCOT, PLOT, packed), their dimensions, applicability and diffusion factors affecting the separation

- Evaluate suitability of common stationary phases for different analytes

- Distinguish types of interactions between the analytes and stationary phase

- Justify the selection of a stationary phase based on its interactions, stability, and bleed with the targeted analyte

- Evaluate and explain the column bleed

Quiz GC Columns

Provide handwritten answers and bring them to the class

- Compare capillary and packed columns, their dimensions, the common number of theoretical plates, and factors affecting peak broadening.

- Explain the principle and strength of dispersive interactions (London forces), dipole-dipole interactions, hydrogen bonding.

- Differentiate stationary phases and discuss their relationship to the interactions with analytes.

- What is the difference in the interactions with analytes on methyl and phenyl dimethyl polysiloxane stationary phases?

- A homological series of alkanes is analyzed using a linear temperature gradient. What defines the elution order? How does tr change for sequentially eluting compounds?

- What are PLOT columns and how are they used?

- What is the column bleed?

- In GC analysis, what is the source of an ion of 207 m/z?

# Stationary Phases and Integrations in GC

Heptanoic Acid C7H14O2

Heptanoic Acid C7H14O2

Glycerol C3H8O3

Glycerol C3H8O3

Heptane C7H16

Heptane C7H16

Match a suitable stationary phase to the following analytes

Nitrobenzene C6H5NO2

Nitrobenzene C6H5NO2

Acetic Acid C2H4O2

Acetic Acid C2H4O2

- List the types of interactions expected between the analytes and stationary phases selected above.

- Which of the stationary phases above would be suitable specifically for GC-MS analysis?

- Which of these stationary phases is the least thermally stable?

# Separation on PLOT columns

- What are PLOT columns? Please sketch.

- Which of the analytes listed below would be separated on a PLOT column?

- Organic solvents

- Polycyclic aromatic hydrocarbons

- Refinery gases

- Air

- Fatty acids

- Which analytes (of those listed above)  would not be suitable for a PLOT column and why?

- Sketch two chromatograms of air on an alumina PLOT column with retention H2< O2 < N2, CO2 using a column with stationary phases of 0.1 µm and 5.0 µm film thickness.

- Would you expect to have longer or shorter retention times on a thicker stationary phase and why?

- How fast would be the elution of the analytes listed above?

- Based on the points 5 and 6 reconsider your chromatograms.

- Would it better work with higher or lower flowrates (Consider Van Deemter theory)

# GC Analysis and Column Bleed

- Sketch a chromatogram of a solvent (hexane) analyzed using the temperature program from 40 °C to 320 °C with a 25 °C/min gradient.

- What is the column bleed? Consider whether it would affect the sketched chromatogram above?

- Sketch three chromatograms and label all peaks of a series of alkanes (C7-C15) dissolved in n-hexane on a dimethyl polysiloxane stationary phase analyzed using a split injection and

  - An Isothermal analysis at 200  °C.

  - A temperature program from 40 °C to 320 °C with a 5 °C/min gradient.

  - A temperature program from 100 °C to 320 °C with a 5 °C/min gradient.

- Do you expect any co-elution? Why?

# Optimizing Temperature Program

- The temperature program and analytes elution profile are shown above.  How could you modify the temperature program to improve the analysis?

- Determine temperatures at which each analyte eluted in the temperature program used above.

- Is your program shorter or longer than the original?

- Based on questions 2 and 3, reconsider the proposed program and suggest a new one.