Calibration Curves

Submit an Excel spreadsheet with your work.

Student Learning Outcomes.  After this exercise, students will be able to:

- Prepare a linear calibration curve from experimental data.

- Perform a linear fit, including regression statistics, in Excel.

- Use a calibration curve to determine the quantity of an unknown.

- Discuss the limitations of a calibration curve.

The purpose of this exercise is to introduce calibration curves in the context of quantitative chemical analysis.

The concentration of chloride in a water sample can be determined by a technique called ion chromatography.  The instrument response is peak area (no units).  A series of standard chloride solutions were prepared by diluting a 30 ppm Cl- standard by 2x, 5x, 10x, and 50x in ultra-pure water.  The peak area of the undiluted and diluted solutions were 6.642, 3.250, 1.308, 0.634, 0.061, respectively.

- Prepare a calibration curve for Cl-.  Perform a linear least squares fit to the data, and display the equation and R2 value on the plot.

- Determine the concentration of Cl- in a sample yielding a peak area of 1.443.

- Determine the uncertainty in the slope and intercept of the calibration curve.

- Five water samples from the same location were analyzed, yielding peak areas of 1.684, 1.454, 1.338, 1.472, and 1.443.

  - Report the concentration of Cl- as the 95% confidence interval.

  - Which source of error – the calibration curve or the sample reproducibility – dominates the total error in the analysis?