Chromatography Basics using HPLC Simulator

Chromatography terms review and introduction to HPLC simulator:

- Go to http://www.multidlc.org/hplcsim (You should open HPLC simulator at the start of each class so that it is available for in-class activities.) The simulator will appear as below.

- Find tR in the data table.

- Click on nitrobenzene to highlight its peak. Note its retention time both in table and on chromatogram:  tR = _____ min.

- In the left menus click on Column Properties where you will find Void time, which is another name for tM:  tM = _____ s = ______ min.

- In the table under the chromatogram, note the k (retention factor) for nitrobenzene:          k = ______.

- Calculate k of nitrobenzene based on tR and void time, and compare it to the value in the table (item 4): k = ______.

Retention control introduction:

- Slide left scroll bar up so that you can see Mobile Phase Composition, and click on it to open.

- Adjust the blue slide bar for Solvent B fraction (%v/v) to 100% acetonitrile.

- What is tR for nitrobenzene?  tR = _____ min.

- What is the void time? (open Column Properties menu at left side of screen, and scroll to bottom to find Void time) tM = _____ s = _____ min.

- Compare the void time at 100% acetonitrile (last question) with that at 40% acetonitrile (question 3 of last exercise = ____min). Justify the values you obtain.

- In the table under the chromatogram, note k (retention factor) for nitrobenzene for 100% acetonitrile: k = ______.

- How does the retention time (tR) and retention factor (k) of nitrobenzene compare to those of the other compounds when using 100% acetonitrile?

8) 	As a group, discuss whether the retention time of a compound can be ever be less than the void time tM?

9) As a group, discuss why the separation of the five compounds is so poor using 100% acetonitrile.

Flow rate introduction:

- Go to http://www.multidlc.org/hplcsim  If simulator was already open, click the Reset button at upper left, beside HPLC Simulator to reset to default conditions (Solvent B Fraction = 40%).

- Click on nitrobenzene in table to highlight this peak.

- Click on Chromatographic Properties to open this menu. Change Flow Rate to 0.5 mL/min. (Need to type in “0.5” rather than use the up/down arrows.)

- What is retention time (tR) of nitrobenzene?  tR =  ________min

- What is retention factor (k) of nitrobenzene?  k  =  _______

- Click on General Properties to open its menu. Uncheck “Automatically determine time span.” Final time should be “763.2” s.

| Flow Rate (mL/min) | tR (min) | k |
| --- | --- | --- |
| 0.5 |  |  |
| 1.0 |  |  |
| 2.0 |  |  |
| 5.0 |  |  |

- Complete the following table for nitrobenzene:

- What is the relationship between flow rate and retention time? Is this reasonable? Explain.

- What is the relationship between flow rate and retention factor? Is this reasonable? Explain.

Retention, Efficiency, and Resolution with HPLC Simulator:

- Go to http://www.multidlc.org/hplcsim  If simulator was already open, return to default conditions using upper left Reset button.

- Open the Mobile Phase Composition menu, and change Solvent B fraction (% v/v) in 1% increments from 40% to 35%. What are the two least resolved compounds in the chromatogram? _____________ and ______________

What happens to the resolution between these two compounds as the Solvent B Fraction is adjusted from 40% to 35% acetonitrile?

- Repeat step 2 noting the retention time tR and standard deviation  for the two least resolved peaks. Four  equals the baseline width of the peak (w = 4). Use the equation below to determine the resolution between these peaks at one flow rate studied in step 3. Each group member should perform the calculation for a unique Solvent B fraction. Show an example calculation. Hint: ensure tR and w are in same units.

Resolution:

Complete the following table for Question 3:

| Solvent B (%) | tR,1 (min) | 1 (s) | w1 (min) | tR,2 (min) | 2 (s) | w2 (min) | Resolution (unitless) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 40 |  |  |  |  |  |  |  |
| 39 |  |  |  |  |  |  |  |
| 38 |  |  |  |  |  |  |  |
| 37 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 35 |  |  |  |  |  |  |  |

- A resolution value of 1.5 or greater is considered baseline resolution. Is baseline resolution achieved with any of these mobile phases? What does baseline resolved mean?

- With Solvent B Fraction (% v/v) of 38% acetonitrile, use retention time tR and  (s) for nitrobenzene to determine plate number N. The width at baseline w equals 4. Other group members should calculate N for 35% acetonitrile.

Plate number:

- Resolution is governed by the Purnell equation.

Purnell equation:	Resolution  =

where N is the plate number, k1 and k2 are the retention factors of the two peaks, and  is the separation factor which equals = k2/k1 where k2  k1. For two different Solvent B Fractions (35% and 38%), record the values of k1 and k2 under the two sets of conditions. Calculate the separation factor (= k2/k1). Calculate the plate number (use N from question 5), separation factor term, and retention factor term of the Purnell equation. Compare with the resolutions determined in question 3.

| Solvent B (%) | k1 | k2 |  | N/4 | (–1)/ | k2/(1+ k2) | Resolution (unitless) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 38 |  |  |  |  |  |  |  |
| 35 |  |  |  |  |  |  |  |
| Change (%) |  |  |  |  |  |  |  |

What term (N, , or k) is most responsible for the improvement in resolution between 38% and 35% acetonitrile? Explain your answer.

- Adjust Solvent B Fraction (% v/v) to 38% acetonitrile. Using Chromatographic Properties to change the flow rate, examine the resolution of the two closest peaks at 0.5, 1.0, 2.0, 3.0, 4.0 and 5.0 mL/min. Which flow rate provides the best resolution?

Optimal flow rate = ________ mL/min

- Using the resolutions determined in questions 2 and 3, guestimate the resolution at one of the flow rates studied in question 7. Each student should guestimate the resolution at a different flow rate.

van Deemter Behavior using HPLC Simulator

Plate height, H, is proportional to the variance of a chromatographic band (i.e., varience = 2, where 4 = baseline width of peak). The smaller the plate height, the narrower the peak.  The van Deemter equation tells us how the column and linear velocity ux affect plate height:

Multiple	Longitudinal	Mass

paths	diffusion	transfer

A, B, and C are constants for a given column and stationary phase.

- Go to http://www.multidlc.org/hplcsim  If simulator was already open, return to default conditions using upper left Reset button. Under Mobile Phase Composition, change Solvent B Fraction to 38% so that benzonitrile and p-chlorophenol are partially resolved. Your chromatogram should appear as below.

- Open the Chromatographic Properties menu, and find H (plate height).

H = _______cm

Plate height H = L/N where L is the column length (found under Column Properties menu) and N is the Plate number (found under Chromatographic Properties). Use L and N to calculate the plate height.

H = ________ mm  =  ________ cm

- Mobile phase velocity in a chromatographic system can be described in a number of different ways. Commonly we use linear velocity = ux = L/tM where L is the column length and tM (also called Void time).

Use the Length and Void time found under the Column Properties menu to determine the linear velocity ux. ux = __________cm/s

Within the Chromatographic Properties menu, which Flow velocity term refers to the linear velocity ux? ___________________________

- Under Chromatographic Properties, adjust Flow Rate from 0.2 to 5 mL/min. At each flow rate, record H (plate height) and Plate number (N) which are reported at the bottom of the Chromatographic Properties menu. Also record the linear velocity ux.

| Flow rate (mL/min) | Linear velocity (cm/s) | N (unitless) | H (cm) |
| --- | --- | --- | --- |
| 0.2 |  |  |  |
| 0.5 |  |  |  |
| 1.0 |  |  |  |
| 2.0 |  |  |  |
| 3.0 |  |  |  |
| 4.0 |  |  |  |
| 5.0 |  |  |  |

- Plot H versus flow rate (or linear velocity) to create a plot of the van Deemter behavior.

- Open the Column Properties menu. Record the values of the van Deemter parameters A, B, and C which are at the bottom of the menu.

A = multiple path broadening = _____

B = longitudinal diffusion broadening = ______

C = finite time for equilibrium (resistance to mass transfer) broadening = _______

- We will now investigate each individual source of band broadening. This is not something that you can do with a real chromatograph. Each group member should select one van Deemter to study: group member A should leave A at its default value in part 6 and type in B = 0.001 and C = 0.001 (The simulator does not accept a value of 0); group member B should leave B at default and type in A = 0.001 and C = 0.001; and group member C should leave C at default, and set A = 0.001 and B = 0.001. Adjust Flow Rate from 0.2 to 5 mL/min, and record N, H and ux.

|  |  | A only |  | B only |  | C only |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Flow rate (mL/min) | Velocity (cm/s) | N | H (cm) | N | H (cm) | N | H (cm) |
| 0.2 |  |  |  |  |  |  |  |
| 0.5 |  |  |  |  |  |  |  |
| 1.0 |  |  |  |  |  |  |  |
| 2.0 |  |  |  |  |  |  |  |
| 3.0 |  |  |  |  |  |  |  |
| 4.0 |  |  |  |  |  |  |  |
| 5.0 |  |  |  |  |  |  |  |

- Plot H versus flow rate (or linear velocity) for the three cases studied in step 7 to show the contribution of each term in the van Deemter equation to the overall peak broadening.

- Explain why each term in the van Deemter plot has a different dependence on the flow rate and linear velocity. Summarize the discussion below.

- Based on the van Deemter plot, state whether the B or C term is more significant at each flow rate or linear velocity.

|  |  | Bigger term? |  |
| --- | --- | --- | --- |
| Flow rate (mL/min) | Velocity (cm/s) | B | C |
| 0.2 |  |  |  |
| 0.5 |  |  |  |
| 1.0 |  |  |  |
| 2.0 |  |  |  |
| 3.0 |  |  |  |
| 4.0 |  |  |  |
| 5.0 |  |  |  |

- What is the significance of the flow rate (linear velocity) at which the dominant contribution to band broadening changed in question 10?

- Return A, B, and C to the default values in question 6. As a group discuss what you think will happen to the optimum flow rate (optimum linear velocity) if the C-term is reduced to 0.02.