Electrodeposition/Electrogravimetry

Will the presence of Fe(II) at 0.0800 M interfere with the electroplating of 99.9% of the cadmium(II) in a solution in which the cadmium is expected to be present at a concentration of no less than 0.0500 M?  Calculate potential values relative to a standard hydrogen electrode.

Suppose the solution had 0.0800 M Cr(III) as a possible interference.  Is it possible to plate out 99.9% of the cadmium(II) without any interference from the chromium.

Coulometry

1.  Draw the plot you would obtain for current (y-axis) versus time (x-axis) if you applied a constant potential high enough to carry the reduction of Cd(II) to cadmium metal.  The system has an electrode with a large surface area.

2.  How would you relate the outcome of your plot above to the concentration of Cd(II) in the sample?

3.  What advantages does coulometry have over electrogravimetry?

Coulometric Titration

What are some advantages of using a coulometric titration?

Amperometric Titration

Draw the plot that would be obtained in an amperometric titration if each of the following occurred (assume that the system is set up with a reducing potential):

- Only the analyte undergoes a reduction at the applied potential.

- Only the titrant undergoes a reduction at the applied potential.

- Both the analyte and titrant undergo a reduction at the applied potential.

Potentiometric Titration

Consider a solution of iron(II) that is to be titrated with cerium(IV).  The products of the reaction are iron(III) and cerium(III).  The course of the titration will be monitored potentiometrically.

Fe(II) + Ce(IV) = Fe(III) + Ce(III)

If 20 ml of an 0.10 M solution of Fe(II) is to be titrated with 0.10 M Ce(IV), calculate the junction potential that would be measured at a platinum electrode when 5 ml, 10 ml, 20 ml, and 30 ml of titrant have been added.

The following steps will allow you to work through the calculation of the junction potentials.

1.  Write an expression for the total electrochemical potential of this system (this is the Nernst equation for the complete reaction).

2.  What is the total electrochemical potential of the titration reaction at any point during the titration?  Think carefully about this because your first intuition may not be correct.

3.  Which half reaction is easier to use to calculate the junction potential before the equivalence point?  Which half reaction is easier to use after the equivalence point?