In-class Questions:  Infrared Spectroscopy

Background information

- Can infrared spectra be recorded in air?  If so, what does this say about the major constituents of air?

- Why don’t the major constituents of air absorb infrared radiation?  It might be worth noting that a molecule such as hydrogen chloride (HCl) does absorb infrared light.

- Describe the vibrations of carbon dioxide (CO2) and determine which ones absorb infrared radiation.

Specialized techniques

- One technique is called non-dispersive infrared (NDIR) spectroscopy.  NDIR is usually used to measure a single constituent of an air sample.  Think what the name implies and consider how such an instrument might be designed.

Fourier-transform Infrared Spectroscopy

- Consider the light path for a Michelson interferometer and plot the intensity of radiation at the sample versus the position of the moveable mirror for monochromatic radiation of wavelength x, 2x or 4x.

- What are the advantages of FT-IR spectrophotometers over conventional IR spectrophotometers that use a monochromator?